# SILABUS LENGKAP: EMBEDDED SYSTEMS ADVANCED (STM32 & FreeRTOS)

# 📘 REFERENSI & BUKU WAJIB

Sebelum masuk ke detail mingguan, berikut adalah referensi utama yang disarankan untuk mendampingi silabus ini:

## Wajib (Primary)

"Mastering STM32" oleh Carmine Noviello - Fokus pada STM32CubeIDE & HAL. Buku ini memberikan panduan komprehensif tentang penggunaan IDE modern dan Hardware Abstraction Layer yang umum dipakai di industri.

"Mastering the FreeRTOS Real Time Kernel" oleh Richard Barry - Dokumentasi resmi FreeRTOS, sangat krusial untuk paruh kedua semester. Ini adalah referensi paling otoritatif untuk Real-Time Operating System.

"The Definitive Guide to ARM Cortex-M3 and Cortex-M4 Processors" oleh Joseph Yiu - Untuk pemahaman arsitektur Low-Level mendalam. Esensial untuk memahami mekanisme internasional prosessor ARM yang digunakan dalam STM32.

## Pendukung (Secondary)

Datasheet & Reference Manual dari MCU target (STM32F103x8 atau ESP32 Technical Reference Manual) - Referensi teknis spesifik hardware yang digunakan.

UM1724 User Manual (STM32 Nucleo boards) atau skematik BluePill - Panduan board development khusus yang akan digunakan dalam praktikum.

# 🗓️ BREAKDOWN MATERI MINGGUAN

# FASE 1: BARE-METAL & HARDWARE ABSTRACTION (Pertemuan 1-6)

Fokus: Memahami bagaimana software mengontrol hardware secara langsung sebelum masuk ke RTOS. Fase ini membangun fondasi yang kuat tentang microcontroller, dari hardware level hingga programming abstraction layer.

## PERTEMUAN 1: Embedded System Lanjutan & Environment Setup

### Tujuan Pembelajaran

1. Memahami arsitektur ARM Cortex-M dan Bus Matrix dalam microcontroller
2. Membedakan konsep Microcontroller vs Microprocessor
3. Menguasai STM32CubeIDE dan Toolchain (GCC, GDB, OpenOCD)
4. Melakukan debugging dasar pada embedded system

### Konten Teori

Arsitektur ARM Cortex-M:
• von Neumann vs Harvard Architecture dalam context STM32
• Memory Map: Flash, RAM, Peripheral regions (0x00000000 - 0xFFFFFFFF)
• Bus Matrix dan AHB/APB bridges untuk komunikasi antar komponen
• Blok diagram internal: CPU core, Memory controller, GPIO, Timer, UART, etc.

Perbedaan Microcontroller vs Microprocessor:
• MCU: Semua komponen (CPU, RAM, Flash, I/O) terintegrasi dalam satu chip
• CPU: Hanya processor core, memerlukan external memory dan I/O
• Trade-off: Power consumption vs Performance
• Aplikasi masing-masing dalam industri

STM32CubeIDE & Toolchain:
• Eclipse-based IDE untuk development STM32
• GCC Compiler: GNU Embedded Toolchain for ARM
• GDB Debugger: Remote debugging protocol
• OpenOCD (Open On-Chip Debugger): Bridge antara IDE dan hardware debugger (ST-Link)
• Project structure yang di-generate oleh CubeMX

### Praktikum

Session 1.1: Environment Installation
• Download dan install STM32CubeIDE (atau SW4STM32 + Cube MX + GCC ARM tools)
• Install J-Link/ST-Link drivers untuk hardware communication
• Setup Eclipse workspace
• Verifikasi compiler version: arm-none-eabi-gcc --version

Session 1.2: Hello World - Blinky LED
• Generate project baru menggunakan CubeMX untuk target board (STM32F103 atau STM32H743)
• Analisis struktur project yang di-generate:
  - Core/Src/main.c: Entry point dan main loop
  - Drivers/HAL: Hardware Abstraction Layer
  - Drivers/CMSIS: Cortex Microcontroller Software Interface Standard
  - Inc/: Header files
  - Linker script: Memory layout definition
• Konfigurasi GPIO untuk LED (PA5 sebagai output)
• Menulis kode sederhana: Toggle LED di dalam main loop
• Build project (Ctrl+B)
• Flash ke board menggunakan ST-Link

Session 1.3: Debugging Fundamentals
• Connect debugger dan jalankan dengan Debug mode (F11)
• Breakpoint: Click pada line number untuk set breakpoint
• Step Over (F10): Execute satu line, tidak masuk fungsi
• Step Into (F11): Masuk ke dalam function call
• Resume (F8): Lanjut eksekusi hingga breakpoint berikutnya
• Variable Watch: Buka Views > Expressions untuk monitor variabel
• Live Expression: Update real-time saat step-by-step
• Examine Memory: Lihat raw memory values di alamat tertentu
• Hands-on: Debug program Blinky, pause di berbagai line, amati nilai register dan memory

### Deliverables

• STM32CubeIDE project yang working dengan Blinky LED
• Screenshot debugging session dengan variable watch
• Dokumentasi singkat: Skematik pin yang digunakan, konfigurasi clock

## PERTEMUAN 2: GPIO Deep Dive (HAL vs Register Level)

### Tujuan Pembelajaran

1. Menguasai semua GPIO mode dan konfigurasinya
2. Memahami perbedaan antara HAL driver dan direct register access
3. Mengimplementasi debouncing untuk input digital
4. Menganalisis trade-off antara abstraksi vs performa

### Konten Teori

GPIO Modes dalam STM32:
• Input Mode:
  - Floating: Input tanpa pull resistor, rentan noise
  - Pull-up: Internal pull-up resistor ke VDD
  - Pull-down: Internal pull-down resistor ke GND
• Output Mode:
  - Push-pull: Output dapat source dan sink current
  - Open-drain: Output hanya bisa sink current (perlu pull-up external)
• Analog: Pin terhubung ke ADC peripheral
• Alternate Function: Pin digunakan oleh peripheral lain (UART TX/RX, SPI, I2C, dsb)

Kecepatan Output (Slew Rate):
• Low (2 MHz): Konsumsi power rendah, risiko EMI rendah
• Medium (10 MHz): Default untuk kebanyakan aplikasi
• High (50 MHz): Untuk komunikasi high-speed
• Very High (100 MHz): Edge rate cepat, EMI tinggi
• Trade-off antara power consumption, EMI, dan signal quality

Konsep Abstraksi dalam Programming:
• Direct Register Access: Menulis langsung ke register hardware (cepat, kompleks)
• Low-Layer (LL) Driver: Thin wrapper, masih close-to-metal
• Hardware Abstraction Layer (HAL): High-level interface, portability tinggi
• Perbandingan: HAL_GPIO_WritePin() vs GPIOA->ODR vs LL_GPIO_SetOutputPin()
• Industri: Kebanyakan produksi menggunakan HAL untuk maintainability

### Praktikum

Session 2.1: Button Input dengan HAL & Software Debouncing
• Setup GPIO button pada input pull-down mode
• Implementasi debouncing algoritma:
```
#define DEBOUNCE_TIME 20  // ms
static uint32_t last_press_time = 0;

void Button_Handler() {
    uint32_t current_time = HAL_GetTick();
    if ((current_time - last_press_time) > DEBOUNCE_TIME) {
        if (HAL_GPIO_ReadPin(GPIOB, GPIO_PIN_0)) {
            LED_Toggle();
            last_press_time = current_time;
        }
    }
}
```
• Test dalam main loop: Polling setiap 10ms

Session 2.2: LED Control dengan HAL Driver
• Toggle LED: HAL_GPIO_TogglePin(GPIOA, GPIO_PIN_5)
• Set LED ON: HAL_GPIO_WritePin(GPIOA, GPIO_PIN_5, GPIO_PIN_SET)
• Set LED OFF: HAL_GPIO_WritePin(GPIOA, GPIO_PIN_5, GPIO_PIN_RESET)
• Kontrol multiple LED: Buat fungsi untuk mengontrol LED states
• Timing measurement: Gunakan debugger untuk measure toggle frequency

Session 2.3: Direct Register Access - Comparison Study
• Implementasi GPIO control menggunakan pointer manipulation:
```
// Define GPIO port base addresses
#define GPIOA_BASE 0x40010800
#define ODR_OFFSET 0x0C

volatile uint32_t *gpioa_odr = (uint32_t*)(GPIOA_BASE + ODR_OFFSET);

// Set PA5 (bit 5)
*gpioa_odr |= (1 << 5);  // Set
*gpioa_odr &= ~(1 << 5); // Reset
```
• Implementasi hal kiri di atas mode LL driver approach
• Benchmark: Mengukur instruction count untuk toggle operation
• Analisis: HAL abstraction cost vs register direct access
• Kesimpulan: Kapan pakai mana, trade-off dalam industri

### Deliverables

• Project dengan button input + debouncing yang working
• Comparison code: Tiga versi implementasi (HAL, LL, Direct Register)
• Benchmark report: Instruction count, execution time, code size
• Test report: Debounce effectiveness dengan button jitter simulation

## PERTEMUAN 3: Clock System & SysTick Timer

### Tujuan Pembelajaran

1. Memahami sistem clock RCC dan oscillator types
2. Mengkonfigurasi PLL untuk mencapai frekuensi maksimum MCU
3. Menggunakan SysTick timer sebagai system tick
4. Mengimplementasi delay function yang presisi tanpa blocking

### Konten Teori

Reset and Clock Control (RCC):
• Bertanggung jawab untuk clock generation, distribution, dan reset
• Clock tree management untuk berbagai peripheral
• Enable/Disable peripheral clock untuk power saving
• Derivasi clock untuk berbagai peripheral (APB1, APB2, AHB)

Oscillator Types:
• HSI (High Speed Internal): On-chip RC oscillator, ~8MHz, less accurate
• HSE (High Speed External): External crystal oscillator, 4-25MHz (typical 8MHz), more accurate
• LSI (Low Speed Internal): On-chip RC oscillator, ~32kHz, untuk low-power apps
• LSE (Low Speed External): External crystal oscillator, 32.768kHz, untuk Real-Time Clock
• Accuracy trade-off: Accuracy vs Power consumption vs Cost

PLL (Phase Locked Loop) Configuration:
• Menggunakan HSE sebagai input (biasanya 8MHz)
• Formula: FOUT = FIN × (PLL_MUL / PLL_DIV)
• Untuk STM32F103: FOUT = 8MHz × 9 = 72MHz (maximum)
• Untuk STM32H743: FOUT bisa hingga 480MHz
• Trade-off: Higher frequency = More power consumption, Heat
• Critical: Voltage regulator harus support target frequency

SysTick Timer:
• 24-bit countdown timer, bagian dari ARM Cortex-M core
• Biasanya dikonfigurasi untuk interrupt setiap 1ms (di FreeRTOS menjadi OS tick)
• Formula: SysTick_Value = (Clock_Speed / Desired_Frequency) - 1
• Untuk 72MHz dengan interrupt 1kHz: SysTick = 72000 - 1
• digunakan untuk: Timing, delay functions, OS scheduler tick
• HAL_Delay() menggunakan SysTick internally

### Praktikum

Session 3.1: Clock Configuration via CubeMX
• Buka CubeMX tool (atau dalam CubeIDE)
• Clock Configuration tab
• Select HSE sebagai clock source
• Configure PLL:
  - Input: HSE 8MHz
  - Multiplier: 9 (untuk 72MHz)
  - Target AHB frequency: 72MHz
• Verify: Clock tree display menunjukkan semua peripheral frequencies
• Generate code dan observe stm32f1xx_hal_conf.c

Session 3.2: Custom delay_ms() menggunakan SysTick
• Implement non-blocking delay tanpa HAL_Delay():
```
#include "stm32f1xx_hal.h"

volatile static uint32_t systick_counter = 0;

void SysTick_Handler(void) {
    systick_counter++;
}

void delay_ms(uint32_t ms) {
    uint32_t start_time = systick_counter;
    while ((systick_counter - start_time) < ms);
}

// Alternative: Busy-wait dengan SysTick reload register
void delay_ms_precise(uint32_t ms) {
    SysTick->LOAD = (SystemCoreClock / 1000) - 1;
    SysTick->VAL = 0;
    SysTick->CTRL = SysTick_CTRL_ENABLE_Msk;
    
    for(uint32_t i = 0; i < ms; i++) {
        while((SysTick->CTRL & SysTick_CTRL_COUNTFLAG_Msk) == 0);
    }
    
    SysTick->CTRL = 0;
}
```
• Test: LED blink dengan delay presisi 500ms

Session 3.3: Observe Clock Configuration Failure
• Konfigurasi HSI saja (tanpa HSE/PLL): Clock = 8MHz
• Jalankan Blinky LED dan measure dengan oscilloscope/logic analyzer
• Toggle frequency: seharusnya ~1kHz untuk 500ms delay
• Kemudian enable PLL ke 72MHz: Frequency seharusnya ~9kHz
• Dokumentasi: Perbedaan behavior dan timing accuracy

### Deliverables

• CubeMX project dengan clock optimized to 72MHz
• Custom delay_ms() implementation dengan documentation
• Clock configuration comparison: HSI vs HSE+PLL
• Measurement report: Actual frequencies vs configured values

## PERTEMUAN 4: Timer & PWM Advanced

### Tujuan Pembelajaran

1. Menguasai konfigurasi General Purpose Timer dan Advanced Timer
2. Memahami Prescaler dan Auto-Reload Register
3. Mengimplementasi PWM untuk kontrol analog output
4. Membuat LED fading dan servo control

### Konten Teori

Timer Types dalam STM32:
• General Purpose Timer (TIM2-TIM5): 16/32-bit, basic timer functions
• Advanced Timer (TIM1, TIM8): 16-bit, dengan complementary output (PWM untuk 3-phase)
• Basic Timer (TIM6-TIM7): 16-bit, untuk interrupt/counting saja (tidak ada I/O)
• Istimewa: TIM9-TIM14 pada beberapa variants
• Pilihan: Tergantung kebutuhan peripheral pins dan frekuensi

Prescaler (PSC) dan Auto-Reload Register (ARR):
• Prescaler: Membagi clock input (formula: Timer_Frequency = Clock_Frequency / (PSC + 1))
• ARR: Definisikan periode timer counting (Timer_Period = (ARR + 1) / Timer_Frequency)
• Contoh: Clock = 72MHz, PSC = 71 → Timer_Frequency = 1MHz
  - Jika ARR = 999 → Period = 1000us = 1ms
• Resolution vs Range: Smaller PSC = finer resolution, smaller max period
• Formula umum: Period(ms) = (ARR + 1) × (PSC + 1) / Clock_Freq × 1000

PWM Generation (Pulse Width Modulation):
• Duty Cycle: Ratio antara ON time vs Total period (0-100%)
• Frequency: 1/Period (dalam Hz)
• CCR (Capture Compare Register): Menentukan duty cycle (0 = 0%, CCR = ARR/2 = 50%, CCR = ARR = 100%)
• PWM Mode 1: Output high saat counter < CCR, low saat counter >= CCR
• PWM Mode 2: Inverse dari Mode 1
• Polarity: Normal atau Inverted
• Aplikasi: LED brightness, motor speed, servo position

### Praktikum

Session 4.1: Timer Interrupt untuk Precise LED Blink
• Configure TIM2 dengan interrupt:
  - PSC = 71 (untuk timer frequency = 1MHz)
  - ARR = 999999 (untuk 1 second period)
  - Enable TIM2 update interrupt
• Implement TIM2_IRQHandler():
```
volatile uint32_t led_toggle_counter = 0;

void TIM2_IRQHandler(void) {
    if (TIM2->SR & TIM_SR_UIF) {
        TIM2->SR &= ~TIM_SR_UIF;  // Clear interrupt flag
        led_toggle_counter++;
        if (led_toggle_counter % 1 == 0) {
            HAL_GPIO_TogglePin(GPIOA, GPIO_PIN_5);
            led_toggle_counter = 0;
        }
    }
}
```
• Test: LED should blink exactly every 1 second (measure dengan logic analyzer)

Session 4.2: PWM LED Brightness Control (Fading Effect)
• Configure TIM3 Channel 1 untuk PWM:
  - PSC = 71 (1MHz timer frequency)
  - ARR = 99 (100 steps, ~10kHz PWM frequency)
  - Channel 1 PWM mode 1
• Implement LED fading di main loop:
```
#define PWM_STEPS 100
#define FADE_SPEED 10  // ms per step

uint16_t pwm_value = 0;
uint8_t direction = 1;  // 1 = up, 0 = down

while(1) {
    HAL_TIM_PWM_Start(&htim3, TIM_CHANNEL_1);
    __HAL_TIM_SET_COMPARE(&htim3, TIM_CHANNEL_1, pwm_value);
    
    HAL_Delay(FADE_SPEED);
    
    if (direction) {
        pwm_value++;
        if (pwm_value >= PWM_STEPS) direction = 0;
    } else {
        pwm_value--;
        if (pwm_value == 0) direction = 1;
    }
}
```
• Observe pada oscilloscope: Duty cycle should vary from 0% to 100% smoothly

Session 4.3: Servo Motor Control (Optional - if hardware available)
• Servo standard: 50Hz frequency, 1-2ms pulse width (1ms = 0°, 1.5ms = 90°, 2ms = 180°)
• Configure TIM4 Channel 1:
  - PSC = 71
  - ARR = 1439 (50Hz frequency: 1MHz / 1440 = 694.4 ≈ 50Hz)
  - Channel 1 PWM mode 1
• Implement servo positioning:
```
#define SERVO_MIN 28   // 1ms pulse (ARR/50 = 28)
#define SERVO_MAX 56   // 2ms pulse (ARR/25 = 56)
#define SERVO_CENTER 42  // 1.5ms pulse

void Set_Servo_Angle(uint8_t angle) {  // angle: 0-180
    uint16_t pulse = SERVO_MIN + (angle * (SERVO_MAX - SERVO_MIN)) / 180;
    __HAL_TIM_SET_COMPARE(&htim4, TIM_CHANNEL_1, pulse);
}
```
• Test: Set servo ke 0°, 90°, 180° dan measure pulse width

### Deliverables

• Project dengan timer interrupt LED blink presisi
• PWM LED fading dengan smooth transitions
• Servo control code (jika hardware tersedia)
• Oscilloscope/Logic Analyzer measurements dokumentasi
• Comparison: Timer interrupt vs software delay untuk precision

## PERTEMUAN 5: Interrupt (NVIC) & DMA

### Tujuan Pembelajaran

1. Menguasai NVIC (Nested Vectored Interrupt Controller)
2. Mengimplementasi External Interrupt (EXTI)
3. Memahami Direct Memory Access dan aplikasinya
4. Mengganti polling dengan interrupt-driven programming

### Konten Teori

NVIC (Nested Vectored Interrupt Controller):
• Bagian dari ARM Cortex-M core, mengatur interrupt handling
• Priority Levels: 0 (highest) to 255 (lowest) - pada STM32F103 hanya 4 bits (0-15)
• Preemption: Interrupt dengan priority tinggi bisa interrupt yang priority rendah
• Sub-Priority: Jika priority sama, urutan eksekusi ditentukan sub-priority
• Priority grouping: Mengalokasikan bit untuk preemption vs sub-priority
• Vector table: Setiap interrupt punya handler function di memory

External Interrupt (EXTI):
• Line 0-15: Connected ke GPIO pins (PA0-PF15)
• Tiap line bisa configure untuk rising edge, falling edge, atau both
• EXTI Controller: Menangkap edge dan generate interrupt request ke NVIC
• Line 16: Connected ke PVD (Power Voltage Detector)
• Line 17: Connected ke RTC Alarm
• Lines 18-23: Reserved untuk internal events
• Software interrupt: EXTI juga bisa trigger dari software (EXTI_SWIER)

DMA (Direct Memory Access):
• Hardware block yang transfer data antar memory atau peripheral tanpa CPU involvement
• Channels: Setiap peripheral bisa punya multiple DMA channel options
• Modes: Memory-to-Memory, Peripheral-to-Memory, Memory-to-Peripheral
• Data width: Byte (8-bit), Half-word (16-bit), Word (32-bit)
• Increment modes: Address dapat auto-increment setelah setiap transfer
• Keuntungan: CPU bisa mengerjakan task lain saat DMA transfer
• Contoh: UART RX/TX menggunakan DMA untuk circular buffer

### Praktikum

Session 5.1: External Interrupt (EXTI) untuk Button
• Configure GPIO button pada PB0 dengan EXTI
• Rising edge trigger (button press)
• Enable EXTI0 interrupt di NVIC
• Implement EXTI0_IRQHandler():
```
void EXTI0_IRQHandler(void) {
    if (EXTI->PR & EXTI_PR_PR0) {
        EXTI->PR = EXTI_PR_PR0;  // Clear flag
        HAL_GPIO_TogglePin(GPIOA, GPIO_PIN_5);  // LED toggle
    }
}
```
• Atau menggunakan HAL wrapper:
```
void HAL_GPIO_EXTI_Callback(uint16_t GPIO_Pin) {
    if (GPIO_Pin == GPIO_PIN_0) {
        HAL_GPIO_TogglePin(GPIOA, GPIO_PIN_5);
    }
}
```
• Advantage vs polling: CPU tidak buang waktu check button state

Session 5.2: DMA Memory-to-Memory Transfer
• Setup DMA1 Channel 1 untuk mem-to-mem:
  - Source: Array A (RAM)
  - Destination: Array B (RAM)
  - Data size: 32-bit (word)
  - Count: 1024 elements
• Implement:
```
// Source dan destination array
uint32_t source_array[1024];
uint32_t dest_array[1024];

void Setup_DMA() {
    DMA_HandleTypeDef hdma;
    
    hdma.Instance = DMA1_Channel1;
    hdma.Init.Direction = DMA_MEMORY_TO_MEMORY;
    hdma.Init.PeriphInc = DMA_PINC_ENABLE;
    hdma.Init.MemInc = DMA_MINC_ENABLE;
    hdma.Init.PeriphDataAlignment = DMA_PDATAALIGN_WORD;
    hdma.Init.MemDataAlignment = DMA_MDATAALIGN_WORD;
    hdma.Init.Mode = DMA_NORMAL;
    hdma.Init.Priority = DMA_PRIORITY_HIGH;
    
    HAL_DMA_Init(&hdma);
    HAL_DMA_Start(&hdma, (uint32_t)source_array, (uint32_t)dest_array, 1024);
}
```
• Benchmark: Measure transfer time vs CPU copy

Session 5.3: UART dengan DMA (Circular Buffer)
• Setup UART + DMA untuk reception:
  - DMA mode: Circular
  - Peripheral: UART RX
  - Destination: Circular buffer (ring buffer)
• Implement circular buffer:
```
#define RX_BUFFER_SIZE 256
uint8_t rx_buffer[RX_BUFFER_SIZE];
uint16_t rx_head = 0;

void HAL_UART_RxCpltCallback(UART_HandleTypeDef *huart) {
    rx_head = RX_BUFFER_SIZE - __HAL_DMA_GET_COUNTER(huart->hdmarx);
}

uint8_t Get_Received_Byte(void) {
    if (rx_head > 0) {
        rx_head--;
        return rx_buffer[rx_head];
    }
    return 0;
}
```
• Test: Send data via UART, receive menggunakan DMA tanpa blocking

### Deliverables

• Button control menggunakan EXTI interrupt
• DMA memory-to-memory transfer implementation
• UART DMA circular buffer dengan test data
• Performance comparison: Polling vs Interrupt vs DMA
• Documentation: Interrupt priority configuration dan trade-offs

## PERTEMUAN 6: Communication Protocol Advanced (UART, I2C, SPI)

### Tujuan Pembelajaran

1. Menguasai UART dengan DMA dan interrupt-driven
2. Mengimplementasi I2C master mode dan device scanning
3. Memahami SPI protocol dan timing
4. Troubleshoot komunikasi serial dan I2C

### Konten Teori

UART (Universal Asynchronous Receiver Transmitter):
• Baudrate: Data transmission speed (9600, 115200 bps typical)
• Parity: None, Even, Odd - untuk error detection
• Stop bit: 1 atau 2 bits menandakan end of frame
• Data bits: 8 atau 9 bits
• Frame: Start bit + Data bits + Parity (optional) + Stop bit(s)
• Issues pada blocking UART:
  - HAL_UART_Transmit() blocks CPU saat wait for TX complete
  - HAL_UART_Receive() blocks CPU saat waiting untuk data
  - Solusi: DMA atau interrupt-driven approach

I2C (Inter-Integrated Circuit):
• Master/Slave architecture
• 7-bit atau 10-bit addressing (mostly 7-bit)
• Two lines: SCL (clock), SDA (data)
• Open-drain output dengan pull-up resistors
• ACK/NACK: Slave respond dengan acknowledge
• Multi-master support (collision detection)
• Kecepatan: Standard (100kHz), Fast (400kHz), Fast-Plus (1MHz)
• Device scanning: Master query semua address (0-127) untuk discover devices

SPI (Serial Peripheral Interface):
• Master/Slave(s) synchronous protocol
• 4 lines: SCLK (clock), MOSI (Master Out Slave In), MISO (Master In Slave Out), SS (Slave Select)
• Full-duplex: Simultaneous send dan receive
• CPOL (Clock Polarity): Idle state (0=low, 1=high)
• CPHA (Clock Phase): Sample at leading atau trailing edge
• Speed: Dapat mencapai puluhan MHz (tergantung slave device)
• Advantages: Higher speed, simple protocol, multiple slave support

### Praktikum

Session 6.1: UART DMA Interrupt-driven Communication
• Configure UART1 + DMA:
  - TX: Memory to Peripheral (DMA1 Channel 4)
  - RX: Peripheral to Memory (DMA1 Channel 5), Circular mode
• Implement non-blocking UART functions:
```
#define UART_RX_BUFFER_SIZE 512

typedef struct {
    uint8_t rx_buffer[UART_RX_BUFFER_SIZE];
    uint16_t rx_head;
    uint16_t rx_tail;
    uint8_t tx_busy;
} UART_HandleDMA;

UART_HandleDMA uart1;

void UART_Send_DMA(const uint8_t *data, uint16_t length) {
    uart1.tx_busy = 1;
    HAL_UART_Transmit_DMA(&huart1, (uint8_t*)data, length);
}

void HAL_UART_TxCpltCallback(UART_HandleTypeDef *huart) {
    uart1.tx_busy = 0;
}

uint16_t UART_Get_Received_Data(uint8_t *buffer, uint16_t max_len) {
    uint16_t current_tail = UART_RX_BUFFER_SIZE - __HAL_DMA_GET_COUNTER(&hdma_usart1_rx);
    uint16_t available = (current_tail >= uart1.rx_tail) ? 
        (current_tail - uart1.rx_tail) : 
        (UART_RX_BUFFER_SIZE - uart1.rx_tail + current_tail);
    
    uint16_t to_copy = (available < max_len) ? available : max_len;
    for (uint16_t i = 0; i < to_copy; i++) {
        buffer[i] = uart1.rx_buffer[uart1.rx_tail];
        uart1.rx_tail = (uart1.rx_tail + 1) % UART_RX_BUFFER_SIZE;
    }
    return to_copy;
}
```
• Test: Send/receive string dengan DMA, measure CPU usage

Session 6.2: I2C Device Scanning (I2C Bus Scanner)
• Configure I2C1 master mode
• Implement address scanner:
```
#define I2C_MAX_DEVICES 128

void I2C_Scan_Devices(uint8_t *found_devices, uint16_t *device_count) {
    *device_count = 0;
    
    for (uint8_t addr = 1; addr < 127; addr++) {
        // Try to communicate dengan address
        if (HAL_I2C_IsDeviceReady(&hi2c1, addr << 1, 1, 100) == HAL_OK) {
            found_devices[*device_count] = addr;
            (*device_count)++;
        }
    }
}

// Alternative: Manual approach
void I2C_Scan_Manual(void) {
    uint8_t data;
    for (uint8_t addr = 0; addr < 128; addr++) {
        if (HAL_I2C_Master_Receive(&hi2c1, addr << 1, &data, 1, 100) == HAL_OK) {
            printf("Device found at address: 0x%02X\\n", addr);
        }
    }
}
```
• Test: Connect I2C LCD module atau DS1307 RTC, scan and find address

Session 6.3: I2C Temperature Sensor (DS1621 atau LM75)
• Write/read register dari I2C sensor:
```
#define DS1621_ADDR 0x48  // 7-bit address
#define TEMP_REGISTER 0x01

// Read temperature
uint16_t I2C_Read_Temperature(void) {
    uint8_t cmd = TEMP_REGISTER;
    uint8_t temp_raw[2];
    
    HAL_I2C_Master_Transmit(&hi2c1, DS1621_ADDR << 1, &cmd, 1, 100);
    HAL_I2C_Master_Receive(&hi2c1, DS1621_ADDR << 1, temp_raw, 2, 100);
    
    int16_t temp = ((int16_t)temp_raw[0] << 8) | temp_raw[1];
    return (temp >> 7);  // Convert ke degree
}

// Write configuration
void I2C_Configure_Sensor(void) {
    uint8_t cmd[2] = {0x01, 0x04};  // CONFIG register, enable continuous mode
    HAL_I2C_Master_Transmit(&hi2c1, DS1621_ADDR << 1, cmd, 2, 100);
}
```
• Test: Read dan display temperature setiap 1 second

Session 6.4: SPI Flash Memory Communication (Optional)
• Configure SPI1 untuk komunikasi dengan flash chip (W25Q80 atau similar)
• Implement SPI commands:
```
#define CMD_READ_ID 0x90
#define CMD_READ_DATA 0x03

void SPI_Read_Device_ID(void) {
    uint8_t tx_data[4] = {CMD_READ_ID, 0, 0, 0};
    uint8_t rx_data[4] = {0};
    
    HAL_GPIO_WritePin(GPIOA, CS_PIN, GPIO_PIN_RESET);  // Pull CS low
    HAL_SPI_TransmitReceive(&hspi1, tx_data, rx_data, 4, 100);
    HAL_GPIO_WritePin(GPIOA, CS_PIN, GPIO_PIN_SET);    // Pull CS high
    
    printf("Manufacturer: 0x%02X, Device: 0x%02X\\n", rx_data[2], rx_data[3]);
}

void SPI_Read_Data(uint32_t address, uint8_t *data, uint16_t length) {
    uint8_t cmd[4] = {CMD_READ_DATA, 
                      (address >> 16) & 0xFF,
                      (address >> 8) & 0xFF,
                      address & 0xFF};
    
    HAL_GPIO_WritePin(GPIOA, CS_PIN, GPIO_PIN_RESET);
    HAL_SPI_Transmit(&hspi1, cmd, 4, 100);
    HAL_SPI_Receive(&hspi1, data, length, 100);
    HAL_GPIO_WritePin(GPIOA, CS_PIN, GPIO_PIN_SET);
}
```
• Test: Read flash ID dan verify komunikasi

### Deliverables

• UART DMA implementation dengan non-blocking TX/RX
• I2C device scanner working code
• I2C sensor driver (temperature reading)
• SPI communication test code (jika ada hardware)
• Communication protocol troubleshooting guide
• Performance analysis: DMA vs Interrupt vs Polling

## PERTEMUAN 7: UJIAN TENGAH SEMESTER (Bare-Metal Hands-On)

### Format Ujian

Tipe: Practical Hands-On Coding (Individual)
Durasi: 3 jam
Materi: Semua topik Bare-Metal (Pertemuan 1-6)
Alat: STM32 development board, debugger, laptop dengan STM32CubeIDE

### Contoh Soal Ujian Tengah Semester

SOAL: "Buat sistem penghitung mundur (countdown timer) yang ditampilkan di Serial Monitor dengan fitur:
1. Inisialisasi timer 60 detik
2. Tampilkan countdown setiap 1 detik di UART (format: 'Time remaining: XX seconds')
3. Tombol A untuk PAUSE/RESUME countdown
4. Tombol B untuk RESET ke 60 detik
5. LED merah menyala saat countdown, LED hijau menyala saat mencapai 0
6. Kecepatan countdown dapat diubah melalui UART command (command format: 'SPEED:x' dimana x adalah divisor)
7. Gunakan Timer interrupt untuk presisi 1 detik
8. Gunakan EXTI interrupt untuk tombol (jangan polling)
9. Gunakan DMA untuk UART transmission

Persyaratan Teknis:
• Must use HAL drivers
• Must implement timer interrupt
• Must implement EXTI for button
• Must use UART DMA
• Code harus clean dan well-commented
• Debugging dengan breakpoint harus demonstrate knowledge"

### Kriteria Penilaian

• Functionality: 40% - Apakah semua requirement working correctly
• Code Quality: 20% - Clean code, well-commented, proper naming
• Hardware Usage: 20% - Correct use of HAL, interrupt, DMA
• Debugging Skill: 10% - Ability to use debugger effectively
• Documentation: 10% - Explanation dan flowchart jika diperlukan

### Tips untuk Ujian

1. Baca soal dengan teliti dan pahami semua requirement
2. Mulai dengan membuat flowchart atau state diagram
3. Implementasi feature satu per satu, test setiap feature
4. Gunakan debugger untuk troubleshoot
5. Jangan lupa initialize semua peripheral via CubeMX/code
6. Test UART output dengan Serial Monitor (PuTTY atau CubeIDE terminal)
7. Verify interrupt trigger dengan debugger breakpoint
8. Leave time untuk code cleanup dan documentation

# FASE 2: REAL-TIME OPERATING SYSTEM / FreeRTOS (Pertemuan 8-14)

Fokus: Multitasking, Deterministic behavior, dan Resource Management. Fase kedua ini mengajarkan bagaimana mengorganisir bare-metal knowledge menjadi RTOS architecture untuk aplikasi yang lebih kompleks dan scalable.

## PERTEMUAN 8: RTOS Concept & Task Management

### Tujuan Pembelajaran

1. Memahami kapan dan mengapa menggunakan RTOS
2. Menguasai kernel, scheduler, dan context switching
3. Mengimplementasi dan mengelola task dengan prioritas
4. Menganalisis starvation dan task interaction issues

### Konten Teori

Super Loop vs RTOS Architecture:
• Super Loop: Single main loop, baca input, process, output (simple, tapi tidak scalable)
  - Disadvantage: Hard-real-time tidak bisa dijamin, difficult untuk multiple periodic tasks
  - Advantage: Predictable, easy to debug, minimal overhead
• RTOS: Multiple independent tasks, kernel manage scheduling
  - Advantage: Scalable, each task can prioritized, deterministic
  - Disadvantage: Complexity, overhead, debugging lebih sulit
• Kapan pakai: RTOS ketika aplikasi memiliki multiple concurrent activities dengan berbeda timing constraints

RTOS Kernel Components:
• Scheduler: Menentukan task mana yang running pada setiap saat
• Context Switcher: Save state task yang sedang run, restore state task berikutnya
• Task Manager: Create, delete, suspend task
• Synchronization Primitives: Mutex, semaphore, queue untuk inter-task communication
• Timer/Clock: Generate OS tick untuk scheduling decision

Scheduling Strategies:
• Preemptive: Kernel dapat interrupt task saat expired time quantum (fairness, deterministic)
• Cooperative: Task sendiri yield control ke kernel (less overhead, tapi unfair)
• Priority-based: Task dengan priority tinggi run terlebih dahulu
• Round-robin: Task dengan priority sama run in turn dengan fixed time quantum

Task States:
• Running: Task sedang execute
• Ready: Task siap run tapi waiting untuk CPU time
• Blocked: Task waiting untuk event (timer, semaphore, I/O)
• Suspended: Task di-suspend oleh kernel atau lainnya
• Deleted: Task dihapus dari scheduler

### Praktikum

Session 8.1: FreeRTOS Porting ke STM32
• Download FreeRTOS kernel source
• Option 1 (Recommended): Gunakan CubeMX untuk generate FreeRTOS project
  - Di CubeMX: Middleware > FreeRTOS
  - Select FreeRTOS (V10.x.x atau latest)
  - Configure heap size (8KB minimum untuk ini demo)
  - Generate project
• Option 2 (Manual): Download kernel, integrate ke existing project
  - Copy FreeRTOS source files
  - Configure FreeRTOSConfig.h dengan project parameters
  - Update linker script jika perlu
• Verify: Project should build successfully

Session 8.2: Create Multiple Tasks dengan Berbeda Prioritas
• Implement tiga tasks:
```
#include "FreeRTOS.h"
#include "task.h"

// Task 1: LED blink cepat (priority: 2)
void Task_Blink_Fast(void *pvParameters) {
    while(1) {
        HAL_GPIO_TogglePin(GPIOA, GPIO_PIN_5);  // Red LED
        vTaskDelay(pdMS_TO_TICKS(100));  // Delay 100ms
    }
}

// Task 2: LED blink lambat (priority: 1)
void Task_Blink_Slow(void *pvParameters) {
    while(1) {
        HAL_GPIO_TogglePin(GPIOB, GPIO_PIN_0);  // Green LED
        vTaskDelay(pdMS_TO_TICKS(500));  // Delay 500ms
    }
}

// Task 3: Serial output (priority: 3 - highest)
void Task_Monitor(void *pvParameters) {
    while(1) {
        printf("System is running...\\n");
        vTaskDelay(pdMS_TO_TICKS(1000));  // Delay 1 second
    }
}

// Create tasks di main()
int main(void) {
    HAL_Init();
    SystemClock_Config();
    MX_GPIO_Init();
    MX_USART1_UART_Init();
    
    xTaskCreate(Task_Blink_Fast, "BlinkerFast", 128, NULL, 2, NULL);
    xTaskCreate(Task_Blink_Slow, "BlinkerSlow", 128, NULL, 1, NULL);
    xTaskCreate(Task_Monitor, "Monitor", 256, NULL, 3, NULL);
    
    vTaskStartScheduler();
    
    // Should never reach here
    while(1);
}
```
• Test: All three LEDs should blink independently dengan correct frequencies

Session 8.3: Demonstrate Task Starvation Problem
• Modify Task_Blink_Fast untuk tidak pernah yield (infinite loop tanpa vTaskDelay):
```
void Task_High_Priority_Hog(void *pvParameters) {
    while(1) {
        // Do nothing, never yield
        // Ini akan prevent lower priority tasks dari running!
    }
}
```
• Observe: Low priority task tidak pernah run
• Explanation: RTOS kernel cannot preempt task jika using cooperative scheduling
• Solusi: Always include vTaskDelay atau blocking call di task loop
• Lesson: Proper task design dengan guaranteed yield points

### Deliverables

• FreeRTOS project successfully ported ke STM32
• Three working tasks dengan correct priority scheduling
• Demonstration dari starvation problem dan solution
• Task state diagram dengan explanation
• Performance metrics: Task execution time, context switch overhead

## PERTEMUAN 9: RTOS Task Interaction & Queue

### Tujuan Pembelajaran

1. Memahami inter-task communication patterns
2. Menguasai queue untuk passing data antar tasks
3. Mengimplementasi producer-consumer pattern
4. Troubleshoot queue deadlock dan overflow

### Konten Teori

Queue dalam FreeRTOS:
• FIFO buffer: First In First Out data structure
• Pass by Value: Data dicopy ke queue (safe, tapi overhead untuk large data)
• Pass by Reference: Pointer dicopy (fast, tapi sender harus maintain memory)
• Blocking: Task bisa block pada xQueueSend atau xQueueReceive
• Timeout: Can specify maximum wait time
• Multiple writers/readers: Queue thread-safe

Inter-task Communication Patterns:
• Producer-Consumer: Satu task produce data, task lain consume
• Request-Response: Task A request, Task B process, response kembali
• Broadcasting: Satu task send ke multiple task melalui event group atau multiple queues
• Pipeline: Data flow through multiple processing stages

### Praktikum

Session 9.1: Producer-Consumer dengan Queue
```
#include "FreeRTOS.h"
#include "task.h"
#include "queue.h"

#define QUEUE_LENGTH 10
#define QUEUE_ITEM_SIZE sizeof(uint32_t)

QueueHandle_t xQueue = NULL;

// Producer Task: Generate random numbers
void Task_Producer(void *pvParameters) {
    uint32_t value = 0;
    
    while(1) {
        value++;
        
        // Send ke queue dengan timeout 100ms
        if (xQueueSend(xQueue, &value, pdMS_TO_TICKS(100)) == pdPASS) {
            printf("Producer sent: %lu\\n", value);
        } else {
            printf("Queue full, producer blocked!\\n");
        }
        
        vTaskDelay(pdMS_TO_TICKS(500));
    }
}

// Consumer Task: Process data dari queue
void Task_Consumer(void *pvParameters) {
    uint32_t received_value;
    
    while(1) {
        // Receive dari queue dengan block indefinitely
        if (xQueueReceive(xQueue, &received_value, portMAX_DELAY) == pdPASS) {
            printf("Consumer received: %lu\\n", received_value);
            HAL_GPIO_TogglePin(GPIOA, GPIO_PIN_5);  // Toggle LED saat receive
            vTaskDelay(pdMS_TO_TICKS(200));  // Simulate processing
        }
    }
}

int main(void) {
    // Initialize...
    
    // Create queue
    xQueue = xQueueCreate(QUEUE_LENGTH, QUEUE_ITEM_SIZE);
    
    if (xQueue == NULL) {
        printf("Failed to create queue!\\n");
        while(1);
    }
    
    // Create tasks
    xTaskCreate(Task_Producer, "Producer", 256, NULL, 2, NULL);
    xTaskCreate(Task_Consumer, "Consumer", 256, NULL, 1, NULL);
    
    vTaskStartScheduler();
    
    while(1);
}
```
• Test: Producer should send, consumer should receive, monitor dengan serial output

Session 9.2: Multiple Producers to Single Consumer
• Modify contoh: Two producers, one consumer
```
void Task_Producer1(void *pvParameters) {
    for(uint32_t i = 0; i < 100; i++) {
        xQueueSend(xQueue, &i, portMAX_DELAY);
        printf("Producer 1: %lu\\n", i);
        vTaskDelay(pdMS_TO_TICKS(200));
    }
    vTaskDelete(NULL);
}

void Task_Producer2(void *pvParameters) {
    for(uint32_t i = 1000; i < 1100; i++) {
        xQueueSend(xQueue, &i, portMAX_DELAY);
        printf("Producer 2: %lu\\n", i);
        vTaskDelay(pdMS_TO_TICKS(300));
    }
    vTaskDelete(NULL);
}
```
• Observe: Consumer correctly receive dari both producers (interleaved)

Session 9.3: Sensor Reading & Processing Pipeline
• Implement three-stage pipeline:
  1. Sensor Reader Task: Read ADC value every 100ms
  2. Filter/Process Task: Receive dari queue, apply moving average
  3. Display Task: Receive dari queue, print hasil
```
typedef struct {
    uint16_t adc_value;
    uint32_t timestamp;
} SensorData_t;

QueueHandle_t xQueueRaw, xQueueProcessed;

void Task_SensorReader(void *pvParameters) {
    SensorData_t data;
    while(1) {
        data.adc_value = ADC_Read();  // Simulated ADC read
        data.timestamp = xTaskGetTickCount();
        xQueueSend(xQueueRaw, &data, portMAX_DELAY);
        vTaskDelay(pdMS_TO_TICKS(100));
    }
}

void Task_Filter(void *pvParameters) {
    SensorData_t raw_data, filtered_data;
    static uint16_t buffer[10];
    static uint8_t index = 0;
    
    while(1) {
        if (xQueueReceive(xQueueRaw, &raw_data, portMAX_DELAY)) {
            buffer[index] = raw_data.adc_value;
            index = (index + 1) % 10;
            
            // Calculate moving average
            uint32_t sum = 0;
            for(int i = 0; i < 10; i++) sum += buffer[i];
            filtered_data.adc_value = sum / 10;
            filtered_data.timestamp = raw_data.timestamp;
            
            xQueueSend(xQueueProcessed, &filtered_data, portMAX_DELAY);
        }
    }
}

void Task_Display(void *pvParameters) {
    SensorData_t data;
    while(1) {
        if (xQueueReceive(xQueueProcessed, &data, portMAX_DELAY)) {
            printf("Sensor (avg): %u at %lu ms\\n", data.adc_value, data.timestamp);
        }
    }
}
```
• Test: Verify end-to-end data flow dengan correct processing

### Deliverables

• Producer-consumer implementation dengan testing
• Multiple producers scenario working correctly
• Three-stage pipeline dengan sensor data processing
• Queue configuration documentation (size, item type, timeout strategy)
• Performance analysis: Throughput, latency, queue utilization

## PERTEMUAN 10: RTOS Semaphore & Mutex

### Tujuan Pembelajaran

1. Memahami race condition dan shared resource problems
2. Menggunakan semaphore untuk synchronization
3. Mengimplementasi mutex untuk mutual exclusion
4. Mengidentifikasi dan resolve priority inversion

### Konten Teori

Race Condition:
• Terjadi ketika multiple tasks mengakses shared resource secara concurrent
• Output tidak predictable tergantung scheduling
• Contoh: Dua task menulis ke UART pada waktu bersamaan → text berantakan
• Solusi: Synchronization mechanism (mutex, semaphore)

Binary Semaphore:
• Counter dengan nilai 0 atau 1
• xSemaphoreGive(): Increment counter, wake one waiting task
• xSemaphoreTake(): Decrement counter, block if zero
• Use case: Signaling antar tasks (e.g., wait for event)

Counting Semaphore:
• Counter dengan nilai 0 hingga max
• xSemaphoreGive(): Increment counter
• xSemaphoreTake(): Decrement counter
• Use case: Resource management dengan multiple copies (e.g., buffer pool)

Mutex (Mutual Exclusion):
• Binary semaphore dengan ownership tracking
• xSemaphoreTakeRecursive(): Task yang same bisa take multiple kali
• xSemaphoreGiveRecursive(): Corresponding give untuk balance take
• Priority Inheritance: Task yang hold mutex naik priority-nya ke waiting task
• Use case: Protect critical sections (e.g., UART access)

Priority Inversion Problem:
• Terjadi ketika low-priority task hold mutex, high-priority task menunggu
• Result: High-priority task di-block oleh low-priority task
• Severity: Dapat break real-time guarantees
• Solusi: Priority inheritance (mutex automatically raise low-priority task priority)
• Alternative: Priority ceiling protocol

### Praktikum

Session 10.1: Demonstrate Race Condition Problem
• Create two tasks yang both menulis ke UART:
```
void Task_Writer1(void *pvParameters) {
    while(1) {
        printf("Task1: Hello from Task 1\\n");
        vTaskDelay(pdMS_TO_TICKS(200));
    }
}

void Task_Writer2(void *pvParameters) {
    while(1) {
        printf("Task2: Hello from Task 2\\n");
        vTaskDelay(pdMS_TO_TICKS(300));
    }
}

// Create both tasks dengan same priority
xTaskCreate(Task_Writer1, "Writer1", 256, NULL, 1, NULL);
xTaskCreate(Task_Writer2, "Writer2", 256, NULL, 1, NULL);
```
• Observe: Output akan garbled/interleaved (race condition terlihat)
• Explanation: Both tasks write tanpa synchronization

Session 10.2: Solve dengan Mutex
• Protect UART access dengan mutex:
```
SemaphoreHandle_t xUART_Mutex = NULL;

void Task_Writer1(void *pvParameters) {
    while(1) {
        if (xSemaphoreTake(xUART_Mutex, portMAX_DELAY)) {
            printf("Task1: Hello from Task 1\\n");
            xSemaphoreGive(xUART_Mutex);
        }
        vTaskDelay(pdMS_TO_TICKS(200));
    }
}

void Task_Writer2(void *pvParameters) {
    while(1) {
        if (xSemaphoreTake(xUART_Mutex, portMAX_DELAY)) {
            printf("Task2: Hello from Task 2\\n");
            xSemaphoreGive(xUART_Mutex);
        }
        vTaskDelay(pdMS_TO_TICKS(300));
    }
}

// Create mutex
xUART_Mutex = xSemaphoreCreateMutex();

xTaskCreate(Task_Writer1, "Writer1", 256, NULL, 1, NULL);
xTaskCreate(Task_Writer2, "Writer2", 256, NULL, 1, NULL);
```
• Test: Output should clean, no interleaving

Session 10.3: Semaphore untuk Interrupt Deferred Handling
• Use binary semaphore untuk signal dari interrupt ke task:
```
SemaphoreHandle_t xButtonSemaphore = NULL;

void EXTI0_IRQHandler(void) {
    BaseType_t xHigherPriorityTaskWoken = pdFALSE;
    
    if (EXTI->PR & EXTI_PR_PR0) {
        EXTI->PR = EXTI_PR_PR0;
        
        // Give semaphore dari ISR
        xSemaphoreGiveFromISR(xButtonSemaphore, &xHigherPriorityTaskWoken);
        
        // Yield jika higher priority task woken
        portYIELD_FROM_ISR(xHigherPriorityTaskWoken);
    }
}

void Task_Button_Handler(void *pvParameters) {
    while(1) {
        if (xSemaphoreTake(xButtonSemaphore, portMAX_DELAY)) {
            printf("Button pressed!\\n");
            HAL_GPIO_TogglePin(GPIOA, GPIO_PIN_5);
        }
    }
}

// Create semaphore
xButtonSemaphore = xSemaphoreCreateBinary();

xTaskCreate(Task_Button_Handler, "ButtonHandler", 256, NULL, 2, NULL);
```
• Advantage vs direct ISR handling: Real work done di task context, ISR quick

### Deliverables

• Race condition demonstration dengan visual evidence
• Mutex-protected critical section implementation
• Semaphore untuk interrupt deferred handling
• Priority inversion analysis dan solution
• Comparison: Performance with/without synchronization

## PERTEMUAN 11: RTOS Timer & Event Group

### Tujuan Pembelajaran

1. Menggunakan software timer dalam RTOS context
2. Mengimplementasi event group untuk synchronization
3. Mengatasi timing-related bugs
4. Membuat application dengan multiple periodic events

### Konten Teori

Software Timer:
• Timer yang dikelola oleh kernel (bukan hardware timer)
• Implement via daemon task yang handle timer expiry
• One-shot: Timer fire sekali saja
• Auto-reload: Timer restart otomatis setelah fire
• Callback: Function yang dipanggil saat timer expire
• Resolution: Limited oleh OS tick frequency (typically 1ms)

Event Group:
• 24-bit flag untuk synchronization (STM32 biasanya 32-bit tapi FreeRTOS gunakan 24)
• Setiap bit represent satu event
• xEventGroupSetBits(): Set bit(s)
• xEventGroupWaitBits(): Wait untuk specific bit(s) set
• Can use AND/OR logic untuk wait kombinasi events
• Useful untuk state machines dengan multiple events

### Praktikum

Session 11.1: Software Timer untuk Periodic Sensor Reading
```
#include "FreeRTOS.h"
#include "timers.h"

TimerHandle_t xSensorTimer = NULL;

void Sensor_Timer_Callback(TimerHandle_t xTimer) {
    // Ini dipanggil setiap 100ms
    uint16_t adc_value = ADC_Read();
    printf("Sensor reading: %u\\n", adc_value);
}

int main(void) {
    // Initialize...
    
    // Create timer: 100ms periode, auto-reload
    xSensorTimer = xTimerCreate("SensorTimer",
                                 pdMS_TO_TICKS(100),  // Period
                                 pdTRUE,               // Auto-reload
                                 0,                    // Timer ID
                                 Sensor_Timer_Callback);
    
    if (xSensorTimer == NULL) {
        printf("Failed to create timer!\\n");
    }
    
    // Start timer
    xTimerStart(xSensorTimer, portMAX_DELAY);
    
    vTaskStartScheduler();
}
```
• Test: Sensor reading setiap 100ms presisi

Session 11.2: Event Group untuk System State Management
• Implement system state dengan multiple events:
```
#define EVENT_WIFI_READY     (1 << 0)   // Bit 0
#define EVENT_SENSOR_READY   (1 << 1)   // Bit 1
#define EVENT_STORAGE_READY  (1 << 2)   // Bit 2
#define ALL_SYSTEMS_READY    (EVENT_WIFI_READY | EVENT_SENSOR_READY | EVENT_STORAGE_READY)

EventGroupHandle_t xSystemReady = NULL;

void Task_WiFi_Init(void *pvParameters) {
    // Simulate WiFi initialization
    printf("Initializing WiFi...\\n");
    vTaskDelay(pdMS_TO_TICKS(1000));
    printf("WiFi ready!\\n");
    xEventGroupSetBits(xSystemReady, EVENT_WIFI_READY);
    vTaskDelete(NULL);
}

void Task_Sensor_Init(void *pvParameters) {
    printf("Initializing Sensor...\\n");
    vTaskDelay(pdMS_TO_TICKS(500));
    printf("Sensor ready!\\n");
    xEventGroupSetBits(xSystemReady, EVENT_SENSOR_READY);
    vTaskDelete(NULL);
}

void Task_Storage_Init(void *pvParameters) {
    printf("Initializing Storage...\\n");
    vTaskDelay(pdMS_TO_TICKS(800));
    printf("Storage ready!\\n");
    xEventGroupSetBits(xSystemReady, EVENT_STORAGE_READY);
    vTaskDelete(NULL);
}

void Task_Main(void *pvParameters) {
    // Wait for all systems ready
    EventBits_t xBits = xEventGroupWaitBits(xSystemReady,
                                            ALL_SYSTEMS_READY,
                                            pdFALSE,  // Don't clear bits
                                            pdTRUE,   // Wait untuk ALL
                                            portMAX_DELAY);
    
    if ((xBits & ALL_SYSTEMS_READY) == ALL_SYSTEMS_READY) {
        printf("All systems ready! Starting main application...\\n");
        while(1) {
            printf("Application running...\\n");
            vTaskDelay(pdMS_TO_TICKS(1000));
        }
    }
}

int main(void) {
    // Initialize...
    
    xSystemReady = xEventGroupCreate();
    
    xTaskCreate(Task_WiFi_Init, "WiFiInit", 256, NULL, 1, NULL);
    xTaskCreate(Task_Sensor_Init, "SensorInit", 256, NULL, 1, NULL);
    xTaskCreate(Task_Storage_Init, "StorageInit", 256, NULL, 1, NULL);
    xTaskCreate(Task_Main, "Main", 256, NULL, 2, NULL);
    
    vTaskStartScheduler();
}
```
• Test: Main task waits untuk semua init selesai

### Deliverables

• Software timer implementation untuk periodic sensor reading
• Event group untuk system state synchronization
• Complex state machine menggunakan event group
• Timing analysis: Accuracy vs OS tick resolution
• Comparison: Software timer vs hardware timer

## PERTEMUAN 12: Low Power Mode & Reliability (Watchdog)

### Tujuan Pembelajaran

1. Memahami STM32 low power modes dan kapan menggunakannya
2. Mengimplementasi watchdog timer untuk sistem reliability
3. Mengintegrasikan low power dengan RTOS idle task
4. Mengukur dan mengoptimalkan power consumption

### Konten Teori

STM32 Low Power Modes:
• Sleep Mode: CPU clock disabled, peripheral still active (10-100µA current)
• Stop Mode: Voltage regulator low power, oscillator disabled (1-10µA)
• Standby Mode: Only RTC dan backup register active (0.1-1µA)
• Trade-off: Power saving vs wake-up latency
• Entry: Via WFI (Wait For Interrupt) atau WFE (Wait For Event)

Watchdog Timer:
• Independent Watchdog (IWDG): Separate low-speed RC oscillator
• Window Watchdog (WWDG): Connected ke APB1 clock
• Purpose: Reset system jika tidak di-refresh dalam timeout period
• Prevent hang: MCU akan auto-reset jika task tidak responding
• Configuration: Timeout, prescaler

RTOS Idle Task Hook:
• Callback yang dipanggil saat idle task running (no ready task)
• Perfect place untuk masuk ke sleep mode
• Kernel automatically exit sleep saat interrupt
• Benefit: Maximize power saving dengan maintain responsiveness

### Praktikum

Session 12.1: Watchdog Timer Implementation
```
void MX_IWDG_Init(void) {
    hiwdg.Instance = IWDG;
    hiwdg.Init.Prescaler = IWDG_PRESCALER_256;  // Prescaler
    hiwdg.Init.Reload = 0xFFF;                   // Max reload (4095), ~5 seconds
    
    if (HAL_IWDG_Init(&hiwdg) != HAL_OK) {
        printf("IWDG Init failed!\\n");
    }
}

// Refresh watchdog dalam task
void Task_Watchdog_Feeder(void *pvParameters) {
    while(1) {
        HAL_IWDG_Refresh(&hiwdg);  // "Pet the dog"
        printf("Watchdog fed\\n");
        vTaskDelay(pdMS_TO_TICKS(2000));  // Feed every 2 seconds
    }
}

// Jika ini task tidak refresh dalam timeout (5s), MCU reset otomatis
```
• Test: Stop feeder task, observe MCU restart
• Measure: Power behavior sebelum dan sesudah watchdog active

Session 12.2: Idle Hook untuk Sleep Mode
• Configure FreeRTOS sleep hook:
```
#include "FreeRTOS.h"
#include "task.h"

void vApplicationIdleHook(void) {
    // Called saat idle task
    // Enter sleep mode untuk save power
    HAL_PWR_EnterSleepMode(PWR_MAINREGULATOR_ON, PWR_SLEEPENTRY_WFI);
}

// Alternative untuk stop mode:
void vApplicationIdleHook_StopMode(void) {
    __disable_irq();
    if (eGetState() == eSleepMode) {
        __enable_irq();
        // Configure stop mode
        HAL_PWR_EnterSTOPMode(PWR_LOWPOWERREGULATOR_ON, PWR_STOPENTRY_WFI);
    }
    __enable_irq();
}
```
• Advantage: Saat no tasks ready, CPU immediately sleep (save power)

Session 12.3: Power Consumption Measurement
• Setup untuk measure current:
  - Disconnect USB power
  - Insert ammeter di power rail
  - Measure current di berbagai scenarios:
    1. Full CPU active (spinning loop)
    2. One task running dengan vTaskDelay
    3. Multiple tasks dengan sleep mode active
    4. Different sleep modes
• Document measurements dan compare dengan datasheet specification

### Deliverables

• Watchdog timer implementation dengan testing
• RTOS idle hook untuk sleep mode integration
• Power measurement data dan analysis
• Wake-up latency measurement
• Best practices guide untuk low power embedded systems

## PERTEMUAN 13: Embedded System Architecture & Design Patterns

### Tujuan Pembelajaran

1. Merancang scalable embedded system architecture
2. Mengimplementasi Finite State Machine (FSM)
3. Memahami layered architecture dan component design
4. Mengenal coding standards (MISRA-C basics)

### Konten Teori

Finite State Machine (FSM):
• Sistem dengan discrete states dan transitions
• Each state memiliki specific actions dan conditions
• Transitions terjadi berdasarkan events atau conditions
• Benefit: Predictable behavior, easy to reason about, simplify debugging
• Implementation: Switch-case dalam task loop

Layered Architecture:
• Hardware Abstraction Layer (HAL): GPIO, UART, Timer drivers
• Middleware: Low-level abstractions (ring buffer, state machine engine)
• Driver Layer: Device-specific drivers (LCD driver, sensor driver)
• Application Layer: Business logic dan high-level features
• Benefit: Modularity, reusability, testability, maintainability

MISRA-C Guidelines (Overview):
• Directive 4.1: Functions should have a single point of exit
• Rule 2.1: Assembly language should be avoided
• Rule 8.4: All declarations should be local to the most limited scope
• Rule 14.1: Loop counter should be signed integer
• Focus: Reduce undefined behavior, increase safety, improve maintainability

### Praktikum

Session 13.1: FSM untuk State Machine
• Design simple appliance controller (e.g., washing machine):
```
typedef enum {
    STATE_IDLE = 0,
    STATE_FILLING,
    STATE_WASHING,
    STATE_DRAINING,
    STATE_SPINNING
} WashMachine_State_t;

typedef enum {
    EVENT_START = 0,
    EVENT_STOP,
    EVENT_FILL_DONE,
    EVENT_WASH_DONE,
    EVENT_DRAIN_DONE,
    EVENT_SPIN_DONE
} WashMachine_Event_t;

void WashMachine_Task(void *pvParameters) {
    WashMachine_State_t current_state = STATE_IDLE;
    WashMachine_Event_t event;
    
    while(1) {
        // Wait untuk event
        if (xQueueReceive(xEventQueue, &event, pdMS_TO_TICKS(100))) {
            // State machine
            switch(current_state) {
                case STATE_IDLE:
                    if (event == EVENT_START) {
                        printf("Starting fill...\\n");
                        start_fill_pump();
                        current_state = STATE_FILLING;
                    }
                    break;
                    
                case STATE_FILLING:
                    if (event == EVENT_FILL_DONE) {
                        printf("Fill complete, starting wash...\\n");
                        start_wash_motor();
                        current_state = STATE_WASHING;
                    } else if (event == EVENT_STOP) {
                        stop_fill_pump();
                        current_state = STATE_IDLE;
                    }
                    break;
                    
                case STATE_WASHING:
                    if (event == EVENT_WASH_DONE) {
                        printf("Wash complete, draining...\\n");
                        stop_wash_motor();
                        start_drain_pump();
                        current_state = STATE_DRAINING;
                    }
                    break;
                    
                case STATE_DRAINING:
                    if (event == EVENT_DRAIN_DONE) {
                        printf("Draining complete, spinning...\\n");
                        start_spin_motor();
                        current_state = STATE_SPINNING;
                    }
                    break;
                    
                case STATE_SPINNING:
                    if (event == EVENT_SPIN_DONE) {
                        printf("Spin complete, idle...\\n");
                        stop_spin_motor();
                        current_state = STATE_IDLE;
                    }
                    break;
            }
        }
    }
}
```
• Test: Sequence melalui semua states dengan correct events

Session 13.2: Layered Architecture Design
• Organize code structure:
```
/project
  /Core
    /Src
      main.c
      system_init.c
    /Inc
      main.h
      config.h
  /Drivers
    /HAL
      hal_gpio.c
      hal_uart.c
    /Device
      sensor_driver.c
      lcd_driver.c
  /Middleware
    /Utils
      ring_buffer.c
      state_machine.c
    /RTOS
      freertos_config.h
  /Application
    /Tasks
      task_sensor.c
      task_display.c
      task_control.c
    /Main
      application.c
```
• Benefits: Clear separation of concerns, reusable components

### Deliverables

• FSM implementation dengan state diagram visualization
• Layered architecture project structure dengan documentation
• MISRA-C code review checklist
• Design pattern examples: Observer, Factory, Singleton dalam embedded context
• Architecture decision document (ADR)

## PERTEMUAN 14: Final Project Preparation & Troubleshooting

### Tujuan

Asistansi dan mentoring untuk final project, troubleshooting hardware issues, code optimization, dan demo preparation.

### Final Project Requirements

Scope: Embedded system project menggunakan STM32 dan FreeRTOS dengan kompleksitas menengah

Komponen Minimum:
• Minimal 3 sensors atau input devices
• Output control (LED, relay, motor, atau display)
• Communication interface (UART, I2C, atau SPI)
• FreeRTOS dengan minimal 3 concurrent tasks
• Real-time constraints (timing-critical operations)
• State management dengan FSM atau event-based architecture
• Power optimization consideration

Tema Contoh:
• Weather Station: Temperature, Humidity, Pressure sensors → Display + Cloud upload
• Smart Lighting: Motion sensor + Light sensor → LED control + UART logging
• Environmental Monitor: Multi-sensor → LCD display + SD card logging
• Robotic Arm Control: Multiple servo + buttons + feedback sensor
• Home Automation: Multiple sensors → Multiple actuators + wireless communication

### Deliverables

1. Working Prototype:
   • Hardware fully assembled dan tested
   • Firmware complete dan functioning
   • All features implemented dan demonstrated

2. Source Code:
   • Well-organized di layers (HAL, Middleware, Application)
   • Fully documented dengan comments
   • MISRA-C compliant (or justification if not)
   • Clean code dengan proper naming conventions

3. Documentation:
   • Project proposal (goals, requirements, design decisions)
   • Hardware schematic dan BOM (Bill of Materials)
   • Software architecture diagram
   • API documentation untuk main modules
   • Testing report (unit test, integration test, system test)
   • Deployment dan usage guide

4. Presentation:
   • Demo video (~5 minutes) atau live demo
   • Technical presentation (~15 minutes)
   • Q&A session dengan evaluator

### Evaluation Criteria

• Functionality: 35% - Apakah semua requirements terpenuhi dan working correctly
• Code Quality: 20% - Clean code, maintainability, proper use of patterns
• RTOS Usage: 15% - Appropriate use FreeRTOS, task design, synchronization
• Hardware Integration: 15% - Sensor interfacing, communication protocols, reliability
• Documentation: 10% - Clarity, completeness, usability
• Presentation: 5% - Technical communication, demo clarity

### Troubleshooting Guidelines

Common Issues dan Solutions:

1. Sensor Noise/Intermittent Readings:
   • Add pull-up/pull-down resistors
   • Implement software debouncing atau filtering
   • Check PCB grounding dan routing
   • Use bypass capacitors near sensor VCC

2. UART Communication Problems:
   • Verify baudrate mismatch
   • Check pin configuration (TX/RX swapped?)
   • Ensure proper voltage levels (3.3V vs 5V)
   • Use logic analyzer untuk debug timing

3. Task Hanging atau Watchdog Reset:
   • Check untuk deadlock dalam synchronization (mutex, queue)
   • Verify task always has yield point (vTaskDelay atau blocking call)
   • Monitor stack usage dengan uxTaskGetStackHighWaterMark()
   • Add debug trace untuk track task execution

4. FreeRTOS Heap Overflow:
   • Increase configTOTAL_HEAP_SIZE di FreeRTOSConfig.h
   • Check untuk memory leak dalam task
   • Monitor heap usage di runtime

5. Timing Issues:
   • Use hardware timer untuk critical timing (tidak rely pada OS tick)
   • Measure actual timing dengan oscilloscope
   • Check untuk OS tick jitter
   • Consider interrupt latency

### Timeline & Milestones

• Week 1-2: Project proposal submission dan approval
• Week 3-4: Hardware assembly dan basic testing
• Week 5-8: Main development phase (firmware implementation)
• Week 9: Integration testing dan debugging
• Week 10: Documentation finalization
• Week 11: Demo rehearsal dan optimization
• Week 12: Final presentation dan evaluation

### Resources dan Tools

Required:
• STM32CubeIDE (free)
• STM32CubeMX (free)
• OpenOCD + GDB (free)
• Serial monitor (PuTTY, Minicom, atau built-in di IDE)

Recommended:
• Oscilloscope atau logic analyzer (untuk timing verification)
• Multimeter (untuk voltage/current measurement)
• USB TTL converter (untuk UART debugging)
• Breadboard, wires, resistors (for prototyping)

Optional:
• Gitflow version control (GitHub, GitLab)
• Doxygen untuk auto-generate documentation
• CppCheck atau static analysis tools
• Unit test framework (Unity atau CppUTest)